psi_common

Documentation

Content

Table of Contents

1	Introduction	6

1.1	Working Copy Structure	6

1.2	VHDL Libraries	6

1.3	Running Simulations	7

1.4	Contribute to PSI VHDL Libraries	8

1.5	Handshaking Signals	9

1.6	TDM	10

2	Packages	11

2.1	psi_common_array_pkg	11

2.2	psi_common_logic_pkg	11

2.3	psi_common_axi_pkg	11

2.4	psi_common_math_pkg	11

3	Memories	12

3.1	psi_common_sdp_ram	12

3.2	psi_common_sp_ram_be	13

3.3	psi_common_tdp_ram	14

3.4	psi_common_tdp_ram_be	15

4	FIFOs	16

4.1	psi_common_async_fifo	16

4.2	psi_common_sync_fifo	19

5	Clock Crossings	21

5.1	psi_common_pulse_cc	21

5.2	psi_common_simple_cc	23

5.3	psi_common_status_cc	25

5.4	psi_common_sync_cc_n2xn	27

5.5	psi_common_sync_cc_xn2n	28

5.6	psi_common_bit_cc	29

5.7	Other Components that can be used as Clock Crossings	29

6	Timing	30

6.1	psi_common_strobe_generator	30

6.2	psi_common_strobe_divider	31

6.3	psi_common_tickgenerator	32

6.4	psi_common_pulse_shaper	33

6.5	psi_common_pulse_shaper_cfg	34

6.6	psi_common_clk_meas	35

7	Conversions	36

7.1	psi_common_wconv_n2xn	36

7.2	psi_common_wconv_xn2n	38

8	TDM Handling	40

8.1	psi_common_par_tdm	40

8.2	psi_common_tdm_par	41

8.3	psi_common_tdm_par_cfg	42

8.4	psi_common_tdm_mux	44

9	Arbiters	45

9.1	psi_common_arb_priority	45

9.2	psi_common_arb_round_robin	47

10	Interfaces	48

10.1	psi_common_spi_master	48

10.2	psi_common_i2c_master	50

10.3	psi_common_axi_master_simple	55

10.4	psi_common_axi_master_full	65

10.5	psi_common_axi_slave_ipif	70

11	Miscellaneous	74

11.1	psi_common_delay	74

11.2	psi_common_pl_stage	75

11.3	psi_common_multi_pl_stage	76

11.4	psi_common_ping_pong	77

11.5	psi_common_delay_cfg	80

11.6	psi_common_trigger_generator	81

Figures

Figure 1: Working copy structure	6

Figure 2: Handshaking signals	9

Figure 3: psi_common_async_fifo: Architecture	17

Figure 4: psi_common_pulse_cc: handling of pulses	22

Figure 5: psi_common_pulse_cc: alignment of pulses can change	22

Figure 6: psi_common_pulse_cc: handling of resets	22

Figure 7: psi_common_simple_cc: Architecture	24

Figure 8: psi_common_status_cc: Architecture	26

Figure 9: psi_common_strobe_generator: Strobe synchronization	30

Figure 10: psi_common_pulse_shaper: Example waveform	33

Figure 11: psi_common_wconv_n2xn: Example waveform	36

Figure 12: psi_common_wconv_xn2n: Data alignment	38

Figure 13: psi_common_wconv_xn2n: Last-Handling and alignment	38

Figure 14: psi_common_par_tdm: Waveform	40

Figure 15: psi_common_tdm_par: Waveform	41

Figure 16: psi_common_tdm_par_cfg: 3 enabled channels waveform	42

Figure 17: psi_common_tdm_par_cfg: 2 enabled channels waveform	42

Figure 18: psi_common_tdm_mux: Waveform	44

Figure 19: psi_common_arb_priority: Waveform	45

Figure 20: psi_common_arb_priority: Parallel prefix computation (PPC)	46

Figure 21: psi_common_arb_round_robin: Waveform	47

Figure 22: psi_common_spi_master: CPOL and CPHA meaning	48

Figure 23: psi_common_spi_master: Parallel interface signal behavior	49

Figure 24: psi_common_i2c_master: Address only transaction	54

Figure 25: psi_common_axi_master_simple: Block diagram	55

Figure 26: psi_common_axi_master_simple: High latency write	56

Figure 27: psi_common_axi_master_simple: High latency write with delay for second transaction	57

Figure 28: psi_common_axi_master_simple: Low latency write	58

Figure 29: psi_common_axi_master_simple: Low latency write with FIFO prefill	59

Figure 30: psi_common_axi_master_simple: Read transaction	60

Figure 31: psi_common_axi_master_simple: Read transaction, low latency	61

Figure 32: psi_common_axi_master_simple: Read transaction, high latency	62

Figure 33: psi_common_axi_master_full: Block diagram	65

Figure 34: psi_common_axi_master_full: Read transaction	66

Figure 35: psi_common_axi_master_full: Write transaction	67

Figure 36: psi_common_axi_slave_ipif: Register Write	71

Figure 37: psi_common_axi_slave_ipif: Register Read	71

Figure 38: psi_common_axi_slave_ipif: Memory Write	72

Figure 39: psi_common_axi_slave_ipif: Memory Read	72

Figure 40: psi_common_axi_slave_ipif: Write over Register/Memory Boundary	72

Figure 41: psi_common_ping_pong TDM mode	77

Figure 42: psi_common_ping_pong parallel	78

Figure 43: psi_common_delay_cfg: Hold behavior & pseudo-architecture	80

Figure 44 psi_common_trigger_generator digital trigger type selected	82

Figure 45 psi_common_trigger_generator analog trigger type selected	82

# Introduction

The purpose of this library is to provide HDL implementations for commonly used VHDL functionality such as memories, FIFOs and clock crossings.

This document serves as description of the RTL implementation for all components.Tipps & Tricks

## Working Copy Structure

If you just want to use some components out of the psi_common library, no special structure is required and the repository can be used standalone.

If you want to also run simulations and/or modify the library, additional repositories are required (available from the same source as psi_common) and they must be checked out into the folder structure shown in the figure below since the repositories reference each-other relatively.

Figure 1: Working copy structure

It is not necessary but recommended to use the name psi_lib as name for the <Root> folder.

## VHDL Libraries

The PSI VHDL libraries (including psi_common) require all files to be compiled into the same VHDL library.

There are two common ways of using VHDL libraries when using PSI VHDL libraries:

- All files of the project (including project specific sources and PSI VHDL library sources) are compiled into the same library that may have any name. 
In this case PSI library entities and packages are referenced by work.psi_<library>_<xxx> (e.g. work.psi_common_pl_stage or work.psi_common_array_pkg.all).

- All code from PSI VHDL libraries is compiled into a separate VHDL library. It is recommended to use the name psi_lib.
In this case PSI library entities and packages are referenced by psi_lib.psi_<lib>_<xxx> (e.g. psi_lib.psi_common_pl_stage or psi_lib.psi_common_array_pkg.all).

## Running Simulations

### Regression Test

#### Modelsim

To run the regression test, follow the steps below:

- Open Modelsim

- The TCL console, navigate to <Root>/VHDL/psi_common/sim

- Execute the command “source ./run.tcl”

All test benches are executed automatically and at the end of the regression test, the result is reported.

#### GHDL

In order to run the regression tests using GHDL, GHDL must be installed and added to the path variable. Additionally a TCL interpreter must be installed.

To run the regression tests using GHDL, follow the steps below:

- Open the TCL interpreter (usually by running tclsh)

- The TCL console, navigate to <Root>/VHDL/psi_common/sim

- Execute the command “source ./runGhdl.tcl”

All test benches are executed automatically and at the end of the regression test, the result is reported

### Working Interactively

During work on library components, it is important to be able to control simulations interactively. To do so, it is suggested to follow the following flow:

- Open Modelsim

- The TCL console, navigate to <Root>/VHDL/psi_common/sim

- Execute the command “source ./interactive.tcl”

  - This will compile all files and initialize the PSI TCL framework

  - From this point on, all the commands from the PSI TCL framework are available, see documentation of PsiSim

- Most useful commands to recompile and simulate entities selectively are

  - compile_files –contains <string>

  - run_tb –contains <string>

The steps vor GHDL are the same, just in the TCL interpreter shall instead of the Modelsim TCL console.

## Contribute to PSI VHDL Libraries

To contribute to the PSI VHDL libraries, a few rules must be followed:

- Good Code Quality

  - There are not hard guidelines. However, your code shall be readable, understandable, correct and save. In other words: Only good code quality will be accepted.

- Configurability

  - If there are parameters that other users may have to modify at compile-time, provide generics. Only code that is written in a generic way and can easily be reused will be accepted.

- Self checking Test-benches

  - It is mandatory to provide a self-checking test-bench with your code.

  - The test-bench shall cover all features of your code

  - The test-bench shall automatically stop after it is completed (all processes halted, clock-generation stopped). See existing test-benches provided with the library for examples.

  - The test-bench shall only do reports of severity error, failure or even fatal if there is a real problem.

  - If an error occurs, the message reported shall start with “###ERROR###:”. This is required since the regression test script searches for this string in reports.

- Documentation

  - Extend this document with proper documentation of your code.

  - Highlight all documentation changes in feature branches in yellow so they can be found easily when merging back to master.

- New test-benches must be added to theregression test-script

  - Change /sim/config.tcl accordingly

  - Test if the regression test really runs the new test-bench and exits without errors before doing any merge requests.

## Handshaking Signals

### General Information

The PSI library uses the AXI4-Stream handshaking protocol (herein after called AXI-S). Not all entities may implement all optional features of the AXI-S standard (e.g. backpressure may be omitted) but the features available are implemented according to AXI-S standard and follow these rules.

The full AXI-S specification can be downloaded from the ARM homepage:
https://developer.arm.com/docs/ihi0051/a

The most important points of the specification are outlined below.

### Excerpt of the AXI-S Standard

A data transfer takes place during a clock cycle where TVALID and TREADY (if available) are high. The order in which they are asserted does not play any role.

- A master is not permitted to wait until TREADY is asserted before asserting TVALID.

- Once TVALID is asserted it must remain asserted until the handshake occurs.

- A slave is permitted to wait for TVALID to be asserted before asserting the corresponding TREADY.

- If a slave asserts TREADY, it is permitted to de-assert TREADY before TVALID is asserted.

An example an AXI handshaking waveform is given below. All the points where data is actually transferred are marked with dashed lines.

Figure 2: Handshaking signals

### Naming

The naming conventions of the AXI-S standard are not followed strictly. The most common synonyms that can be found within the PSI VHDL libraries are described below:

TDATA	InData, OutData, Data, Sig, Signal, <application specific names>

TVALID	Vld, InVld, OutVld, Valid, str, str_i

TREADY	Rdy, InRdy, OutRdy

Note that instead of one TDATA signal (as specified by AXI-S) the PSI VHDL Library sometimes has multiple data signals that are all related to the same set of handshaking signals. This helps with readability since different data can is represented by different signals instead of just one large vector.

## TDM

Rules

- If multiple signals are transferred TDM (time-division-multiplexed) over the same interface and all signals have the same sample rate, no additional channel indicator is implemented and looping through the channels is implicit (e.g. for a three channel link, samples are transferred in the channel order 0-1-2-0-1-2-…).

- If multiple signals are transferred TDM over the same interface and the channels do not have the same sample rate, an additional channel indicator containing the channel number is present.

Reasoning

Not having an additional channel indicator for the most common TDM use-case of multiple signals at the same sample rate prevents any combinatorial blocks from having to know about being used for a TDM signal and maintaining the channel indicator. This implicitly allows using all combinatorial library elements (e.g. binary divider, function approximations, etc.) also for TDM signals

# Packages

## psi_common_array_pkg

### Description

This package defines various array types that are not defined by VHDL natively. Some of these definitions are no more required in VHDL 2008 but since VHDL 2008 is not yet fully synthesizable, the package is kept.

## psi_common_logic_pkg

### Description

This package contains various logic functions (e.g. combinatorial conversions) that can be synthesized.

## psi_common_axi_pkg

### Description

This package contains record definitions to allow representing a complete AXI interface including all ports by only two records (one in each direction). This helps improving the readability of entities with AXI interfaces.

## psi_common_math_pkg

### Description

This package contains various mathematical functions (e.g. log2). The functions are meant for calculating compile-time constants (i.e. constans, port-widths, etc.). They can potentially be synthesized as combinatorial functions but this is neither guaranteed nor will it lead to optimal results.

# Memories

## psi_common_sdp_ram

### Description

This component implements a simple dual port RAM. It has one write port and one read port and both ports are running at the same clock. The RAM is described in a way that it utilizes RAM resources (Block-RAM and Distributed-RAM) available in FPGAs with commonly used tools.

The RAM is a synchronous RAM, so data is available at the read port one clock cycle after applying the address.

The RAM behavior (read-before-write or write-before-read) can be selected. This allows efficiently implementing RAMs for different technologies (some technologies implement one, some the other behavior).

### Generics

Depth_g		Depth of the memory
Width_g		Width of the memory
IsAsync_g		true = Memory is asynchronous, Clk is used for write, RdClk for read
 			false = Memory is synchronous, Clk is used for read and write
RamStyle_g	“auto”	 (default)	Automatic choice of block-  or distributed-RAM
 			“distributed”		Use distributed RAM (LUT-RAM)
 			“block”		Use block RAM
Behavior_g	“RBW”		Read-before-write implementation
 			“WBR”		Write-before-read implementation

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| RdClk | Input | 1 | Read clock (only used if IsAsync_g = true) |
| Write Port |  |  |  |
| WrAddr | Input | ceil(log2(Depth_g)) | Write address |
| Wr | Input | 1 | Write enable (active high) |
| WrData | Input | Width_g | Write data |
| Read Port |  |  |  |
| RdAddr | Input | ceil(log2(Depth_g)) | Read address |
| Rd | Input | 1 | Read enable (active high) |
| RdData | Output | Width_g | Read data |

## psi_common_sp_ram_be

### Description

This component implements a single port RAM with byte enables. The RAM is described in a way that it utilizes RAM resources (Block-RAM and Distributed-RAM) available in FPGAs with commonly used tools.

The RAM is a synchronous RAM, so data is available at the read port one clock cycle after applying the address.

The RAM behavior (read-before-write or write-before-read) can be selected. This allows efficiently implementing RAMs for different technologies (some technologies implement one, some the other behavior).

### Generics

Depth_g		Depth of the memory
Width_g		Width of the memory in bits (must be a multiple of 8)
Behavior_g	“RBW”		Read-before-write implementation
 			“WBR”		Write-before-read implementation

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Access Port |  |  |  |
| Addr | Input | ceil(log2(Depth_g)) | Access address |
| Wr | Input | 1 | Write enable (active high) |
| Be | Input | Width_g/8 | Byte enables (Be[0] corresponds do Din[7:0]) |
| Din | Input | Width_g | Write data |
| Dout | Output | Width_g | Read data |

## psi_common_tdp_ram

### Description

This component implements a true dual port RAM. It has one write port and one read port and both ports can be running at different clocks (completely asynchronous clocks are allowed). The RAM is described in a way that it utilizes RAM resources (Block-RAM) available in FPGAs with commonly used tools.

The RAM is a synchronous RAM, so data is available at the read port one clock cycle after applying the address.

The RAM behavior (read-before-write or write-before-read) can be selected. This allows efficiently implementing RAMs for different technologies (some technologies implement one, some the other behavior).

### Generics

Depth_g		Depth of the memory
Width_g		Width of the memory
Behavior_g	“RBW”		Read-before-write implementation
 			“WBR”		Write-before-read implementation

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Port A |  |  |  |
| ClkA | Input | 1 | Port A clock |
| AddrA | Input | ceil(log2(Depth_g)) | Port A address |
| WrA | Input | 1 | Port A write enable (active high) |
| DinA | Input | Width_g | Port A write data |
| DoutA | Output | Width_g | Port A read data |
| Port B |  |  |  |
| ClkB | Input | 1 | Port B clock |
| AddrB | Input | ceil(log2(Depth_g)) | Port B address |
| WrB | Input | 1 | Port B write enable (active high) |
| DinB | Input | Width_g | Port B write data |
| DoutB | Output | Width_g | Port B read data |

### Constraints

For the RAM to work correctly, signals from one clock domain to the other must be constrained to have not more delay that one clock cycle of the faster clock.

Example with a 100 MHz clock (10.0 ns period) and a 33.33 MHz clock (30 ns period) for Vivado:

set_max_delay –datapath_only –from <ClkA> -to <ClkB> 10.0 
set_max_delay –datapath_only –from <ClkB> -to <ClkA> 10.0

## psi_common_tdp_ram_be

### Description

Same as 3.3 psi_common_tdp_ram but with byte-enables. A byte is only written if WrX is set and the corresponding BeX bit is set too.

### Generics

Depth_g		Depth of the memory
Width_g		Width of the memory, must be a multiple of 8
Behavior_g	“RBW”		Read-before-write implementation
 			“WBR”		Write-before-read implementation

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Port A |  |  |  |
| ClkA | Input | 1 | Port A clock |
| BeA | Input | Width_g/8 | Port A byte enables |
| AddrA | Input | ceil(log2(Depth_g)) | Port A address |
| WrA | Input | 1 | Port A write enable (active high) |
| DinA | Input | Width_g | Port A write data |
| DoutA | Output | Width_g | Port A read data |
| Port B |  |  |  |
| ClkB | Input | 1 | Port B clock |
| BeB | Input | Width_g/8 | Port B byte enables |
| AddrB | Input | ceil(log2(Depth_g)) | Port B address |
| WrB | Input | 1 | Port B write enable (active high) |
| DinB | Input | Width_g | Port B write data |
| DoutB | Output | Width_g | Port B read data |

### Constraints

Same as 3.3 psi_common_tdp_ram

# FIFOs

## psi_common_async_fifo

### Description

This component implements an asynchronous FIFO (different clocks for write and read port). The memory is described in a way that it utilizes RAM resources (Block-RAM) available in FPGAs with commonly used tools.

The FIFO is a fall-through FIFO and has AXI-S interfaces on read and write side.

The RAM behavior (read-before-write or write-before-read) can be selected. This allows efficiently implementing FIFOs for different technologies (some technologies implement one, some the other behavior).

### Generics

Width_g			Width of the FIFO
Depth_g			Depth of the FIFO
AlmFullOn_g		True = Almost full output is provided, False = Almost full output is omitted
AlmFullLevel_g		Almost full output is high if the level is >= AlmFullLevel_g
AlmEmptyOn_g		True = Almost empty output is provided, False = Almost empty output is omitted
AlmEmptyLevel_g	Almost empty output is high if the level is <= AlmFullLevel_g
RamStyle_g		“auto”	 (default)	Automatic choice of block-  or distributed-RAM
 				“distributed”		Use distributed RAM (LUT-RAM)
 				“block”		Use block RAM
RamBehavior_g		“RBW”		Read-before-write implementation
 				“WBR”		Write-before-read implementation
RdyRstState_g		State of InRdy signal during reset. Usually this does not play a role and the default 
 				setting (‘1’) that leads to the least logic on the InRdy path is fine. Setting the value to 
 				‘0’ may lead to less optimal performance in terms of FMAX.

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| InClk | Input | 1 | Write side clock |
| InRst | Input | 1 | Write side reset input (active high) |
| OutClk | Input | 1 | Read side clock |
| OutRst | Input | 1 | Read side reset input (active high) |
| Input Data (InClk domain) |  |  |  |
| InData | Input | Width_g | Write data |
| InVld | Input | 1 | AXI-S handshaking signal |
| InRdy | Output | 1 | AXI-S handshaking signal |
| Output Data (OutClk domain) |  |  |  |
| OutData | Output | Width_g | Read data |
| OutVld | Output | 1 | AXI-S handshaking signal |
| OutRdy | Input | 1 | AXI-S handshaking signal |
| Input Status (InClk domain) |  |  |  |
| InFull | Output | 1 | FIFO full signal synchronous to InClk |
| InEmpty | Output | 1 | FIFO empty signal synchronous to InClk |
| InAlmFull | Output | 1 | FIFO almost full signal synchronous to InClk Only exists if AlmFullOn_g = true |
| InAlmEmpty | Output | 1 | FIFO almost empty signal synchronous to InClk Only exists if AlmEmptyOn_g = true |
| InLevel | Output | ceil(log2(Depth_g))+1 | FIFO level synchronous to InClk |
| Output Status (OutClk domain) |  |  |  |
| OutFull | Output | 1 | FIFO full signal synchronous to OutClk |
| OutEmpty | Output | 1 | FIFO empty signal synchronous to OutClk |
| OutAlmFull | Output | 1 | FIFO almost full signal synchronous to OutClk Only exists if AlmFullOn_g = true |
| OutAlmEmpty | Output | 1 | FIFO almost empty signal synchronous to OutClk Only exists if AlmEmptyOn_g = true |
| OutLevel | Output | ceil(log2(Depth_g))+1 | FIFO level synchronous to OutClk |

### Architecture

The rough architecture of the FIFO is shown in the figure below. Note that the figure does only depict the general architecture and not each and every detail.

Read and write address counters are handled in their corresponding clock domain. The current address counter value is then transferred to the other clock-domain by converting it to gray code, synchronizing it using a double-stage synchronizer and convert it back to a two’s complement number. This approach ensures that a correct value is received, even if the clock edges are aligned in a way that causes metastability on the first flip-flop. Because the data is transferred in gray code, in this case either the correct value before an increment of the counter or the correct value after the increment is received, so the result is always correct.

All status information is calculated separately in both clock domains to make it available synchronously to both clocks.

This architecture is independent of the FPGA technology used and can also be used to combine more than just one Block-RAM into one big FIFO.

Figure 3: psi_common_async_fifo: Architecture

### Constraints

For the FIFO to work correctly, signals from one clock domain to the other must be constrained to have not more delay that one clock cycle of the faster clock.

Example with a 100 MHz clock (10.0 ns period) and a 33.33 MHz clock (30 ns period) for Vivado:

set_max_delay –datapath_only –from <ClkA> -to <ClkB> 10.0 
set_max_delay –datapath_only –from <ClkB> -to <ClkA> 10.0

## psi_common_sync_fifo

### Description

This component implements a synchronous FIFO (same clock for write and read port). The memory is described in a way that it utilizes RAM resources (Block-RAM or distributed RAM) available in FPGAs with commonly used tools.

The FIFO is a fall-through FIFO and has AXI-S interfaces on read and write side.

The RAM behavior (read-before-write or write-before-read) can be selected. This allows efficiently implementing FIFOs for different technologies (some technologies implement one, some the other behavior).

### Generics

Width_g			Width of the FIFO
Depth_g			Depth of the FIFO
AlmFullOn_g		True = Almost full output is provided, False = Almost full output is omitted
AlmFullLevel_g		Almost full output is high if the level is >= AlmFullLevel_g
AlmEmptyOn_g		True = Almost empty output is provided, False = Almost empty output is omitted
AlmEmptyLevel_g	Almost empty output is high if the level is <= AlmFullLevel_g
RamStyle_g		“auto”	 (default)	Automatic choice of block-  or distributed-RAM
 				“distributed”		Use distributed RAM (LUT-RAM)
 				“block”		Use block RAM
RamBehavior_g		“RBW”		Read-before-write implementation
 				“WBR”		Write-before-read implementation
RdyRstState_g		State of InRdy signal during reset. Usually this does not play a role and the default 
 				setting (‘1’) that leads to the least logic on the InRdy path is fine. Setting the value to 
 				‘0’ may lead to less optimal performance in terms of FMAX.

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Rst | Input | 1 | Reset input (active high) |
| Input Data (InClk domain) |  |  |  |
| InData | Input | Width_g | Write data |
| InVld | Input | 1 | AXI-S handshaking signal |
| InRdy | Output | 1 | AXI-S handshaking signal |
| Output Data (OutClk domain) |  |  |  |
| OutData | Output | Width_g | Read data |
| OutVld | Output | 1 | AXI-S handshaking signal |
| OutRdy | Input | 1 | AXI-S handshaking signal |

| Input Status (InClk domain) |  |  |  |
| --- | --- | --- | --- |
| InFull | Output | 1 | FIFO full signal synchronous to InClk |
| InEmpty | Output | 1 | FIFO empty signal synchronous to InClk |
| InAlmFull | Output | 1 | FIFO almost full signal synchronous to InClk Only exists if AlmFullOn_g = true |
| InAlmEmpty | Output | 1 | FIFO almost empty signal synchronous to InClk Only exists if AlmEmptyOn_g = true |
| InLevel | Output | ceil(log2(Depth_g))+1 | FIFO level synchronous to InClk |
| Output Status (OutClk domain) |  |  |  |
| OutFull | Output | 1 | FIFO full signal synchronous to OutClk |
| OutEmpty | Output | 1 | FIFO empty signal synchronous to OutClk |
| OutAlmFull | Output | 1 | FIFO almost full signal synchronous to OutClk Only exists if AlmFullOn_g = true |
| OutAlmEmpty | Output | 1 | FIFO almost empty signal synchronous to OutClk Only exists if AlmEmptyOn_g = true |
| OutLevel | Output | ceil(log2(Depth_g))+1 | FIFO level synchronous to OutClk |

# Clock Crossings

## psi_common_pulse_cc

### Description

This component implements a clock crossing for transferring single pulses from one clock domain to another (completely asynchronous clocks).

The entity shall only be used for single-cycle pulses and the pulse frequency must be lower than the frequency of the slower clock for it to work correctly.

The entity does only guarantee that all pulses arrive at the destination clock domain. It does not guarantee that pulses that occur in the same clock cycle on the source clock domain, occur on the target clock domain in the same clock cycle. As a result it should only be used to do clock-crossings for individual pulses.

This entity does also do the clock-crossing for the reset by using “asynchronously assert, synchronously de-assert” synchronizer chains and applying all attributes to synthesize them correctly.

### Generics

NumPulses_g			Width of the FIFO

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Clock Domain A |  |  |  |
| ClkA | Input | 1 | Clock A |
| RstInA | Input | 1 | Clock domain A reset input (active high) |
| RstOutA | Output | 1 | Clock domain A reset output (active high) - active if RstInA or RstInB is asserted - de-asserted synchronously to ClkA |
| PulseA | Input | NumPulses_g | Input of the pulse signals |
| Clock Domain B |  |  |  |
| ClkB | Input | 1 | Clock B |
| RstInB | Input | 1 | Clock domain A reset input (active high) |
| RstOutB | Output | 1 | Clock domain B reset output (active high) - active if RstInA or RstInB is asserted - de-asserted synchronously to ClkA |
| PulseB | Output | NumPulses_g | Output of the pulse signals |

### Architecture

The figure below shows how the pulses are transferred from one clock domain to the other.

Figure 4: psi_common_pulse_cc: handling of pulses

Since each pulse is handled separately, the pulse alignment may change because of the clock crossing. This is shown in the figure below.

Figure 5: psi_common_pulse_cc: alignment of pulses can change

The figure below shows how the reset signal is transferred from one clock domain to the other. This concept is used to transfer resets in both directions between the clock domains but for simplicity only one direction is shown in the figure.

Figure 6: psi_common_pulse_cc: handling of resets

### Constraints

This entity does not require any constraints.

## psi_common_simple_cc

### Description

This component implements a clock crossing for transferring single values from one clock domain to another (completely asynchronous clocks). In both clock domains the valid samples are marked with a Vld signal according to the AXI-S specification but back-pressure (Rdy) is not handled.

For the entity to work correctly, the data-rate must be significantly lower (4x lower) than the destination clock frequency.

This entity does also do the clock-crossing for the reset by using “asynchronously assert, synchronously de-assert” synchronizer chains and applying all attributes to synthesize them correctly.

### Generics

Width_g		Width of the data signal to implement the clock crossing for

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Clock Domain A |  |  |  |
| ClkA | Input | 1 | Clock A |
| RstInA | Input | 1 | Clock domain A reset input (active high) |
| RstOutA | Output | 1 | Clock domain A reset output (active high) - active if RstInA or RstInB is asserted - de-asserted synchronously to ClkA |
| DataA | Input | Width_g | Data signal input |
| VldA | Input | 1 | AXI-S handshaking signal |
| Clock Domain B |  |  |  |
| ClkB | Input | 1 | Clock B |
| RstInB | Input | 1 | Clock domain A reset input (active high) |
| RstOutB | Output | 1 | Clock domain B reset output (active high) - active if RstInA or RstInB is asserted - de-asserted synchronously to ClkA |
| DataB | Output | Width_g | Data signal output |
| VldB | Output | 1 | AXI-S handshaking signal |

### Architecture

The concept of this clock crossing is to use psi_common_pulse_cc for the clock crossing of the valid signal and latch the data signal on in both clock domains when it is valid. Since the data signal stays stored on the source clock domain, it if for sure valid when the Vld signal arrives at the destination clock domain.

To ensure the clock crossing works, the next Vld signal is only allowed to arrive after the last one was processed. This is the reason for the maximum data rate allowed being limited to one quarter of the destination clock frequency.

Figure 7: psi_common_simple_cc: Architecture

For details about the reset clock crossing, refer to 5.1.4.

### Constraints

For the entity to work correctly, signals from the source clock domain to the destination clock domain must be constrained to have not more delay that one clock cycle of the destination clock.

Example with a 100 MHz source clock (10.0 ns period) and a 33.33 MHz destination clock (30 ns period) for Vivado:

set_max_delay –datapath_only –from <ClkA> -to <ClkB> 30.0

## psi_common_status_cc

### Description

This component implements a clock crossing for slowly changing status information that does not have exact sample rates. It can for example be used to transfer a buffer fill level from one clock domain to another with minimal effort.

The entity ensures that data from the source clock domain is correctly transferred to the destination clock domain. The value at the destination clock domain is always correct in terms of “either the last transferred value or the next one”. The exact timing of the sampling points at which the data is transferred is generated by the entity itself, so it is unknown to the user. As a result, the entity does not guarantee to show transfer state of the data signal in the source clock domain to the destination clock domain in cases of fast changing signals.

For the entity to work correctly, the data-rate must be significantly lower (10 x lower) than the slower clock frequency. Of course the signal can change more quickly but the clock crossing will skip some values in this case.

This entity does also do the clock-crossing for the reset by using “asynchronously assert, synchronously de-assert” synchronizer chains and applying all attributes to synthesize them correctly.

### Generics

Width_g		Width of the data signal to implement the clock crossing for

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Clock Domain A |  |  |  |
| ClkA | Input | 1 | Clock A |
| RstInA | Input | 1 | Clock domain A reset input (active high) |
| RstOutA | Output | 1 | Clock domain A reset output (active high) - active if RstInA or RstInB is asserted - de-asserted synchronously to ClkA |
| DataA | Input | Width_g | Data signal input |
| Clock Domain B |  |  |  |
| ClkB | Input | 1 | Clock B |
| RstInB | Input | 1 | Clock domain A reset input (active high) |
| RstOutB | Output | 1 | Clock domain B reset output (active high) - active if RstInA or RstInB is asserted - de-asserted synchronously to ClkA |
| DataB | Output | Width_g | Data signal output |

### Architecture

The concept of this clock crossing is to use psi_common_simple_cc for the data signal, so the main functionality of this entity is to automatically generate valid pulses.

The first Vld pulse is generated in clock domain ClkA after the reset. At this point, the data is sampled and transferred to ClkB. When the Vld pulse arrives at ClkB, it is transferred back to ClkA and the next data word is transferred. This setup ensures that Vld pulses are generated at a rate that allows the psi_common_simiple_cc to transfer the data correctly without any knowledge about the frequencies of both clock domains.

The concept is shown in the figure below.

Figure 8: psi_common_status_cc: Architecture

### Constraints

For the entity to work correctly, signals from the source clock domain to the destination clock domain must be constrained to have not more delay that one clock cycle of the destination clock.

Example with a 100 MHz source clock (10.0 ns period) and a 33.33 MHz destination clock (30 ns period) for Vivado:

set_max_delay –datapath_only –from <ClkA> -to <ClkB> 30.0

## psi_common_sync_cc_n2xn

### Description

This component implements a clock crossing with AXI-S handshaking for transferring data from one clock domain to another one that runs at an integer multiple of the frequency of the input clock frequency. It can for example be used to transfer data from a 50 MHz clock domain to a 100 MHz clock domain (both generated by the same PLL).

### Generics

Width_g		Width of the data signal to implement the clock crossing for

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Input |  |  |  |
| InClk | Input | 1 | Input side clock |
| InRst | Input | 1 | Input side reset |
| InVld | Input | 1 | AXI-S handshaking signal |
| InRdy | Output | 1 | AXI-S handshaking signal |
| InData | Input | Width_g | Data signal input |
| Output |  |  |  |
| OutClk | Input | 1 | Output side clock |
| OutRst | Input | 1 | Output side reset |
| OutVld | Output | 1 | AXI-S handshaking signal |
| OutRdy | Input | 1 | AXI-S handshaking signal |
| OutData | Output | Width_g | Data signal output |

### Constraints

Constraints are derived by the tools automatically since the clocks are synchronous. Therefore no user constraints are required.

## psi_common_sync_cc_xn2n

### Description

This component implements a clock crossing with AXI-S handshaking for transferring data from one clock domain to another one that runs at an integer fractional of the frequency of the input clock frequency. It can for example be used to transfer data from a 100 MHz clock domain to a 50 MHz clock domain (both generated by the same PLL).

### Generics

Width_g		Width of the data signal to implement the clock crossing for

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Input |  |  |  |
| InClk | Input | 1 | Input side clock |
| InRst | Input | 1 | Input side reset |
| InVld | Input | 1 | AXI-S handshaking signal |
| InRdy | Output | 1 | AXI-S handshaking signal |
| InData | Input | Width_g | Data signal input |
| Output |  |  |  |
| OutClk | Input | 1 | Output side clock |
| OutRst | Input | 1 | Output side reset |
| OutVld | Output | 1 | AXI-S handshaking signal |
| OutRdy | Input | 1 | AXI-S handshaking signal |
| OutData | Output | Width_g | Data signal output |

### Constraints

Constraints are derived by the tools automatically since the clocks are synchronous. Therefore no user constraints are required.

## psi_common_bit_cc

### Description

This component implements a clock crossing for multiple independent single-bit signals. It contains double-stage synchronizers and sets all the attributes required for proper synthesis.

Note that this clock crossing does not guarantee that all bits arrive in the same clock cycle at the destination clock domain, therefore it can only be used for independent single-bit signals.

### Generics

NumBits_g		Number of data bits to implement the clock crossing for

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Input |  |  |  |
| BitsA | Input | NumBits_g | Input signals |
| Output |  |  |  |
| ClkB | Input | 1 | Destination clock |
| BitsB | Output | NumBits_g | Output signals |

### Constraints

No special constraints are required (only the period of the output clock.

## Other Components that can be used as Clock Crossings

- psi_common_tdp_ram (see 3.3)

- psi_common_async_fifo (see 4.1)

# Timing

## psi_common_strobe_generator

### Description

This component generates a strobe (pulse signal with 1 clock cycle pulse-width) at a compile-time configurable frequency. Clock frequency and strobe frequency can be passed as generics.

Optionally the strobe generation can be synchronized to an external signal.

### Generics

freq_clock_g		Frequency of the clock in Hz
freq_strobe_g		Frequency of the strobe output in Hz
rst_pol_g			Reset polarity (‘1’ = high active)

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| InClk | Input | 1 | Clock |
| InRst | Input | 1 | Reset input |
| InSync | Input | 1 | Synchronization signal (optional) |
| OutVld | Output | 1 | Strobe output |

### Synchronization

The strobe synchronization is optional. If no synchronization is required, sync_i can be left unconnected or tied to ‘0’.

When strobe synchronization is used, the strobe signal is synchronized to rising edges detected on the sync_i input. If a rising edge is detected on the sync_i input, a strobe is generated in the next cycle. From there, the strobe is running at the normal frequency.

The figure below shows the behavior of strobe synchronization for a strobe output at ¼ of the clock frequency.

Figure 9: psi_common_strobe_generator: Strobe synchronization

## psi_common_strobe_divider

### Description

This component divides the rate of a strobe signal. Only every Nth strobe signal is forwarded to the output. If the input is not a single cycle strobe signal, a rising edge detection is done (strobe is detected on the first cycle the input is high).

The division ratio is selectable at runtime.

### Generics

length_g		Width of the ratio_i input in bits
rst_pol_g		Reset polarity (‘1’ = high active)

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| InClk | Input | 1 | Clock |
| InRst | Input | 1 | Reset input |
| InVld | Input | 1 | Strobe input |
| InRatio | Input | length_g | Division ratio (1 = no division, 2 = division by 2) 0 leads to the same behavior as 1. |
| OutVld | Output | 1 | Strobe output |

## psi_common_tickgenerator

### Description

This component generated pulses at the commonly used time bases in a system (second, millisecond, microsecond) based on the clock frequency. The width of the tick-pulses is configurable.

### Generics

g_CLK_IN_MHZ					Clock frequency in MHz
g_TICK_WIDTH					Pulse-width of the tick outputs
g_SIM_SEC_SPEEDUP_FACTOR		Factor to speedup the second tick in simulations (reduction of 
 							simulation runtimes). For implementation this generic must be set 
 							to 1.

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clock_i | Input | 1 | Clock input |
| tick1us_o | Output | 1 | Microsecond tick output |
| tick1ms_o | Output | 1 | Millisecond tick output |
| tick1sec_o | Output | 1 | Second tick output |

## psi_common_pulse_shaper

### Description

This component creates pulses of an exactly known length from pulses with unknown length. Additionally it can limit the maximum pulse rate by applying a hold-off time.

Input pulses are detected based on their rising edge.

The figure below shows an example behavior for Duration_g=3 and HoldOff_g=4. The first pulse is stretched to three cycles, the second pulse is ignored because it is within the hold-off time and the third pulse is shortened to three cycles.

Figure 10: psi_common_pulse_shaper: Example waveform

### Generics

Duration_g		Duration of the output pulse in clock cycles

HoldIn_g		If true it holds the input at the output, in case the pulse isn’t pulse but a start signal HoldOff_g		Minimum time between input pulse-rising-edges that are detected (in clock cycles)
 			Pulses arriving during the hold-off time are ignored.

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Clk | Input | 1 | Clock input |
| Rst | Input | 1 | Reset input |
| InPulse | Input | 1 | Input Pulse |
| OutPulse | Output | 1 | Output Pulse |

## psi_common_pulse_shaper_cfg

### Description

This component is similar to psi_common_pulse_shaper (cd §6.4) but it is configurable in runtime. It allows giving the pulse width (duration) and the hold off time as registers in runtime.

### Generics

MaxDuration_g	Maximum duration allowed in clock cycles

HoldOffEna_g	Enable hold-off mode – skip new pulses if arriving too fast

HoldIn_g		If true it holds the input at the output, in case the pulse isn’t pulse but a start signal

MaxHoldOff_g		Maximum clock cycles for the minimum time between each new input pulse-rising-edges that are detected (in clock cycles) - Pulses arriving during the hold-off time are ignored

RstPol_g		Defines reset polarity

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk_i | Input | 1 | Clock input |
| rst_i | Input | 1 | Reset input, polarity is set by generic |
| dat_i | Input | 1 | Input Pulse |
| width_i | Input | Log2ceil(MaxDuration_g) | Pulse width (duration) in clock cycles, if set to 0 no pulse will be generated. |
| hold_i | Input | Log2ceil(MaxHoldOff_g) | Minimum time between each new input pulse-rising edges that are detected (in clock cycles) – Pulse arriving during hold-off time are ignored |
| dat_o | Output | 1 | Output Pulse |

## psi_common_clk_meas

### Description

This component measures the clock (ClkTest) under the assumption that a second clock (ClkMaster) runs at a known frequency.

### Generics

MasterFrequency_g				Clock frequency of the master clock in Hz
MaxMeasFrequency_g				Maximum supported frequency for ClkTest

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ClkMaster | Input | 1 | Master input clock |
| Rst | Input | 1 | Reset (synchronous to ClkMaster) |
| ClkTest | Input | 1 | Test input clock |
| FrequencyHz | Output | 32 | Clock frequency for ClkTest in Hz |
| FrequencyVld | Output | 1 | Handshaking signal (set on every update of FrequencyHz) |

# Conversions

## psi_common_wconv_n2xn

### Description

This component implements a data width conversion from N-bits to a multiple of N-bits. The sample rate is reduced accordingly. The width conversion implements AXI-S handshaking signals to handle back-pressure.

The with conversion supports back-to-back conversions (InVld can stay high all the time). It also handles the last-flag correctly according to AXI specification. If InLast is asserted, all data is flushed out and the word enabled (OutWe) at the output are set only for words that contain data. OutLast is asserted accordingly.

The entity does little-endian data alignment as shown in the figure below.

Figure 11: psi_common_wconv_n2xn: Example waveform

This entity does only do a width conversion but not clock crossing. If a half-clock-double-width conversion is used, psi_common_sync_cc_xn2n component can be used after the width conversion.

### Generics

InWidth_g		Input data width
OutWidth_g	Output data width

The ratio OutWidth_gInWidth_g must be an integer number and OutWidth_g must be bigger or equal to InWidth_g.

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Rst | Input | 1 | Reset (high active) |
| Input |  |  |  |
| InVld | Input | 1 | AXI-S handshaking signal |
| InRdy | Output | 1 | AXI-S handshaking signal |
| InData | Input | InWidth_g | Data signal input |
| InLast | Input | 1 | AXI-S handshaking signal If InLast is asserted, the data stored inside the with-conversion is flushed out. |
| Output |  |  |  |
| OutVld | Output | 1 | AXI-S handshaking signal |
| OutRdy | Input | 1 | AXI-S handshaking signal |
| OutData | Output | OutWidth_g | Data signal output |
| OutLast | Output | 1 | AXI-S handshaking signal |
| OutWe | Output | OutWidth_g/InWidth_g | Output word-enable. Works like byte-enable but with one bit per input-word. All bits in this signal are set, exept for with conversion results flushed out by InLast=’1’. In this case, the OutWe bits indicate which OutData bits contain valid data. |

## psi_common_wconv_xn2n

### Description

This component implements a data width conversion from a multiple N-bits to a N-bits. The sample rate is increased accordingly. The width conversion implements AXI-S handshaking signals to handle back-pressure.

The width conversion does support back-to-back conversions (OutVld/OutRdy can stay high all the time).

The entity does little-endian data alignment as shown in the figure below.

Figure 12: psi_common_wconv_xn2n: Data alignment

The with conversion does also handle the last-flag according to AXI specification and it can do alignment. To do so, an input word-enable signal InWe exists. Words that are not enabled are not sent to the output. If the input is marked with the InLast flag, the last enabled word is marked with OutLast at the output.

Note that with the assertion of InLast at least one byte of the data must be valid (InWe high). Otherwise it would be unclear when OutLast shall be assigned.

Figure 13: psi_common_wconv_xn2n: Last-Handling and alignment

This entity does only do a width conversion but not clock crossing. If a double-clock-half-width conversion is used, psi_common_sync_cc_n2xn component can be used in front of the width conversion.

### Generics

InWidth_g		Input data width
OutWidth_g	Output data width

The ratio InWidth_gOutWidth_g must be an integer number and InWidth _g must be bigger or equal to OutWidth_g.

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Rst | Input | 1 | Reset (high active) |
| Input |  |  |  |
| InVld | Input | 1 | AXI-S handshaking signal |
| InRdy | Output | 1 | AXI-S handshaking signal |
| InData | Input | InWidth_g | Data signal input |
| InLast | Input | 1 | AXI-S handshaking signal |
| InWe | Input | InWidth_g/OutWidth_g | Output word-enable. Works like byte-enable but with one bit per input-word (not per byte). At least one word must be enabled together with the assertion of InLast |
| Output |  |  |  |
| OutVld | Output | 1 | AXI-S handshaking signal |
| OutRdy | Input | 1 | AXI-S handshaking signal |
| OutData | Output | OutWidth_g | Data signal output |
| OutLast | Output | 1 | AXI-S handshaking signal |

# TDM Handling

## psi_common_par_tdm

### Description

This component changes the representation of multiple channels from parallel to time-division-multiplexed. It does not implement any flow-control, so the user is responsible to not apply input data faster than it can be represented at the output (time-division-multiplexed).

The figure below shows some waveforms of the conversion. The lowest bits of the input vector are interpreted as channel 0 and played out first, the highest bits of the input vector are played out last.

Figure 14: psi_common_par_tdm: Waveform

### Generics

ChannelCount_g		Number of channels
ChannelWidth_g		Number of bits per channel

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Rst | Input | 1 | Reset (high active) |
| Input |  |  |  |
| ParallelVld | Input | 1 | AXI-S handshaking signal |
| Parallel | Input | ChannelCount_g*ChannelWidth_g | Data of all channels in parallel. Channel 0 is in the lowest bit and played out first. |
| Output |  |  |  |
| TdmVld | Output | 1 | AXI-S handshaking signal |
| Tdm | Output | ChannelWidth | Data signal output |

## psi_common_tdm_par

### Description

This component changes the representation of multiple channels from time-division-multiplexed to parallel. It does not implement any flow-control.

The figure below shows some waveforms of the conversion. The first input sample is interpreted as channel 0 and played out in the lowest bits of the output, the last input sample is played out in the highest bits.

Figure 15: psi_common_tdm_par: Waveform

Note that the output stays stable also after the Vld pulse.

### Generics

ChannelCount_g		Number of channels
ChannelWidth_g		Number of bits per channel

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Rst | Input | 1 | Reset (high active) |
| Input |  |  |  |
| TdmVld | Input | 1 | AXI-S handshaking signal |
| Tdm | Input | ChannelWidth | TDM input signal, first sample is channel 0. |
| Output |  |  |  |
| ParallelVld | Output | 1 | AXI-S handshaking signal |
| Parallel | Output | ChannelCount_g*ChannelWidth_g | Data of all channels in parallel. Channel 0 is in the lowest bits. |

## psi_common_tdm_par_cfg

### Description

This component changes the representation of multiple channels from time-division-multiplexed to parallel. It does not implement any flow-control.

The number of enabled channels is configurable. In addition, if used with an AXI stream, TdmLast can be used to ensure the correct correspondence between the channels and their indexes in Parallel when the number of enabled channels is changed.

The figures below show some waveforms of the conversion. The first input sample is interpreted as channel 0 and played out in the lowest bits of the output, the last input sample is played out in the enabled highest bits.

Figure 16: psi_common_tdm_par_cfg: 3 enabled channels waveform

Figure 17: psi_common_tdm_par_cfg: 2 enabled channels waveform

Note that the output stays stable also after the Vld pulse.

### Generics

ChannelCount_g		Number of channels
ChannelWidth_g		Number of bits per channel

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Rst | Input | 1 | Reset (high active) |
| EnabledChannels | Input | ChannelCount_g | Number of enabled output channels |
| Input |  |  |  |
| TdmVld | Input | 1 | AXI-S handshaking signal |
| TdmLast | Input | 1 | AXI-S handshaking signal |
| Tdm | Input | ChannelWidth | TDM input signal, first sample is channel 0. |
| Output |  |  |  |
| ParallelVld | Output | 1 | AXI-S handshaking signal |
| Parallel | Output | ChannelCount_g*ChannelWidth_g | Data of all channels in parallel. Channel 0 is in the lowest bits. |

## psi_common_tdm_mux

### Description

This component allows selecting one unique channel over a bunch of “N” time division multiplexed (tdm) data. The output comes with a strobe/valid signal at the falling edge of the “tdm” strobe/valid input with a two clock cycles latency.

Figure 18: psi_common_tdm_mux: Waveform

### Generics

rst_pol_g			reset polarity selection
num_channel_g		Number of channels
data_length_g		Width of the data signals

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| InClk | Input | 1 | Clock |
| InRst | Input | 1 | Reset |
| Inputs |  |  |  |
| InChSel | Input | Log2ceil(num_channel_g) | Mux select |
| InTdmVld | Input | 1 | Strobe/valid input signal (num_channel_g *clock cycle) |
| InTdmDat | Input | data_length_g | Data input |
| Ouputs |  |  |  |
| OutTdmVld | Output | 1 | AXI-S handshaking signal |
| OutTdmDat | Output | data_length_g | Data output |

# Arbiters

## psi_common_arb_priority

### Description

This entity implements a priority arbiter. The left-most bit (highest bit) of the request vector that was asserted is granted (i.e. asserted in the grant vector). The arbiter is implemented using the very logic- and timing-efficient parallel prefix computation approach.

The arbiter can be implemented with or without an output register. The waveform below shows its implementation without output register (OutputRegister_g = false), since the delay would make the waveform less easy to read.

Figure 19: psi_common_arb_priority: Waveform

### Generics

Size_g			Size of the arbiter (number of input/output bits)
OutputRegister_g	True = Registered output
 				False = Combinatorial output

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Rst | Input | 1 | Reset (high active) |
| Data |  |  |  |
| Request | Input | Size_g | Request input signals The highest (left-most) bit has highest priority |
| Grant | Output | Size_g | Grant output signal |

### Architecture

Parallel prefix computation is used to calculate a vector that contains a ‘1’ on the highest-priority bit that was asserted and on all bits with lower priority. The vector then looks for example like this “0001111”. The bit to assert in the Grant output can then be determined by finding the 0-1 edge inside that vector.

The figure below shows the parallel prefix computation graphically.

Figure 20: psi_common_arb_priority: Parallel prefix computation (PPC)

## psi_common_arb_round_robin

### Description

This entity implements a round-robin arbiter. If multiple bits are asserted in the request vector, the left-most bit is forwarded to the grant vector first. Next, the second left-most bit that is set is forwarded etc. Whenever at least one bit in the Grant vector is asserted, the Grant_Vld handshaking signal is asserted to signal that a request was granted. The consumer of the Grant vector signalizes that the granted access was executed by pulling Grant_Rdy high.

Note that the round-robin arbiter is implemented without an output register. Therefore combinatorial paths between input and output exist and it is recommended to add a register-stage to the output as early as possible.

Figure 21: psi_common_arb_round_robin: Waveform

Especially interesting is the part in orange. At this point the arbiter does not grant access to bit 3 because it already granted this request in the clock cycle before. However, it continues to grant access to the highest-priority (i.e. left-most) bit of the request vector that is still left of the bit set in the last Grant output. If the request vector asserts a higher priority this change is directly forwarded to the output. This is shown in the orange section of the waveform.

### Generics

Size_g			Size of the arbiter (number of input/output bits)

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Rst | Input | 1 | Reset (high active) |
| Data |  |  |  |
| Request | Input | Size_g | Request input signals The highest (left-most) bit has highest priority |
| Grant | Output | Size_g | Grant output signal |
| Grant_Vld | Outpu | 1 | AXI-S handshaking signal Asserted whenever Grant != 0 |
| Grant_Rdy | Input | 1 | AXI-S handshaking signal The state of the arbiter is updated upon Grant_Rdy = ‘1’ |

# Interfaces

## psi_common_spi_master

### Description

This entity implements a simple SPI master. All common SPI settings are parametrizable to ensure the master can be configured for different applications.

The clock and data phase is configurable according to the SPI standard terminology described in the picture below:

Figure 22: psi_common_spi_master: CPOL and CPHA meaning

For CPHA = 1, the sampling happens on the second edge (blue) and data is applied on the first edge (red). For CPHA = 0 it is the opposite way.

### Generics

ClockDivider_g			Ratio between Clk and the SpiSck frequency
TransWidth_g			SPI Transfer width (bits per transfer)
CsHighCycles_g			Minimal number of Cs_n high cycles between two transfers
SpiCPOL_g			SPI clock polarity (see figure above)
SpiCPHA_g			SPI sampling edge configuration (see figure above)
SlaveCnt_g			Number of slaves to support (number of Cs_n lines)
LsbFirst_g				False = MSB first transmission, True = LSB first transmission

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Rst | Input | 1 | Reset (high active) |
| Parallel Interface |  |  |  |
| Start | Input | 1 | A high pulse on this line starts the transfer. Note that starting a transaction is only possible when Busy is low. |
| Slave | Input | log2(SlaveCnt_g) | Slave number to access |
| Busy | Output | 1 | High during a transaction |
| Done | Output | 1 | Pulse that goes high for exactly one clock cycle after a transaction is done and RdData is valid |
| WrData | Input | TransWidth_g | Data to send to slave. Sampled during Start = ‘1’ |
| RdData | Output | TransWidth_g | Data received from slave. Must be sampled during Done = ‘1’ or Busy = ‘0’. |
| SPI Interface |  |  |  |
| SpiSck | Output | 1 | SPI clock |
| SpiMosi | Output | 1 | SPI master to slave data signal |
| SpiMiso | Input | 1 | SPI slave to master data signal |
| SpiCs_n | Output | SlaveCnt_g | SPI slave select signal (low active) |

Figure 23: psi_common_spi_master: Parallel interface signal behavior

## psi_common_i2c_master

### Description

This entity implements an I2C master that is multi-master capable (i.e. supports arbitration). The psi_common_i2c_master allows generating start, stop and repeated start conditions as well as transferring bytes. This entity also supports slaves that do clock-stretching.

Addressing is not handled separately. To send the byte containing the address and the R/W bit, the normal byte transfer is used. The user is responsible for filling the address into bits 7:1 and the R/W flag into bit 0 of the data.

The same applies for 10-bit addressing. The user is responsible to send the 10-bit addressing pattern using two byte transfers.

The bus state (busy or free) is tracked based on start- and stop-conditions. However, if no transactions are ongoing (i.e. SCL = ‘1’ and SDA = ‘1’) for a configurable timeout, the bus is always regarded as free. This helps handling the case where a master starts a transaction and then (e.g. due to reset) aborts the transaction without sending a stop-condition.

The user has three main interfaces:

- The command interface allows the user to select the next bus action to do. Commands are:

  - START		Send a start condition (only allowed if bus is idle)

  - STOP		Send a stop condition (only allowed if bus is not idle)

  - REPSTART	Send a repeated start condition (only allowed if the bus is not idle)

  - SEND		Send a data byte (only allowed if the bus is not idle).
 			The data to send is provided along with the command.

  - REC		Receive a data byte (only allowed if the bus is not idle)
 			The ACK to send is provided along with the command.

- On the response interface, the user receives one response per command as soon as the command is completed. More information like the received ACK bit, whether the command was aborted due to a lost arbitration, the received data or wether the command sequence was illegal is sent along with the response.

- On the status interface the user can see if the I2C bus is currently busy or free and if a command timed out (i.e. the user failed to provide the next command within time, so the bus was released).

For constants, a package psi_common_i2c_master_pkg is defined in the same VHDL file.

### Generics

ClockFrequency_g		Frequency of the clock Clk in Hz
I2cFrequency_g			Frequency of the I2cScl signal in Hz
BusBusyTimeout_g		If I2cScl = ‘1’ and I2cSda = ‘1’ for this time in sec., the bus is regarded as free.
 					If the user does not provide any command for this time, the 
 					psi_common_i2c_master automatically generates a stop-condition to release 
 					the bus.
CmdTimeout_g			When the psi_common_i2c_master is ready for the next command but the user 
 					does not provide a new command, after this timeout the bus is releases (and 
 					TimeoutCmd is pulsed to inform the user).
InternalTriState_g		True = Use internal tri-state buffer (I2cScl and I2cSda) are used.
 					False = Use external tri-state buffer (I2cScl_x and I2cSda_x) are used.
DisableAsserts_g		If true, the psi_common_i2c_master does not print any messages during 
 					simulations.

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Rst | Input | 1 | Reset (high active) |
| Command Interface |  |  |  |
| CmdRdy | Output | 1 | AXI-S handshaking signal for command interface |
| CmdVld | Input | 1 | AXI-S handshaking signal for command interface |
| CmdType | Input | 3 | Constant names “000” => Send start condition (CMD_START) “001” => Send stop condition (CMD_STOP) “010” => Send repeated start condition” (CMD_REPSTART) “011” => Send data byte (CMD_SEND) “100” => Receive data byte (CMD_RECEIVE) |
| CmdData | Input | 8 | Input data to send (only for CMD_SEND resp. CmdType=”011”). |
| CmdAck | Input | 1 | Acknowledge to send (only for CMD_RECEIVE resp. CmdType=”100”). |
| Response Interface |  |  |  |
| RspVld | Output | 1 | AXI-S handshaking signal for response interface |
| RspType | Output | 3 | Type of the command that completed. See CmdType for details. |
| RspData | Output | 8 | Received data (only for CMD_RECEIVE resp. CmdType=”100”). |
| RspAck | Output | 1 | 1 => ACK received 0 => NACK received |
| RspArbLost | Output | 1 | The command failed because arbitration was lost. |
| RspSeq | Output | 1 | The command failed because of wrong command sequence (e.g. attempt to do a CMD_START in the middle of an ongoing transfer) |
| Status Interface |  |  |  |
| BusBusy | Output | 1 | I2C bus is busy (used by this master or another master) |
| TimeoutCmd | Output | 1 | Pulsed if the bus is released due to a timeout. |
| I2C Interface – Internal Tristate (InternalTriState_g=true) |  |  |  |
| I2cScl | Bidir | 1 | SCL signal |
| I2cSda | Bidir | 1 | SDA signal |
| I2C Interface – External Tristate (InternalTriState_g=false) |  |  |  |
| I2cScl_I | Input | 1 | SCL input signal |
| I2cScl_O | Output | 1 | SCL output signal |
| I2cScl_T | Output | 1 | SCL Tri-State signal (1 = tristated, 0 drive) |
| I2cSda_I | Input | 1 | SDA input signal |
| I2cSda_O | Output | 1 | SDA output signal |
| I2cSda_T | Output | 1 | SDA Tri-State signal (1 = tristated, 0 drive) |

### Typical Command Sequences

This section provides a few examples for command/response sequences for typical usecases.

Note that the response of the last command is always available before the next command must be asserted. So it is possible to apply commands based on responses received (e.g. not applying a new command if arbitration was lost).

#### Two Byte Read

- CMD_START – send start condition

- CMD_SEND – send address byte (slave responds with ACK)

- CMD_REC – receive data and send ACK

- CMD_REC – receive data and send NACK

- CMD_STOP – send stop condition

| Order | CmdType | CmdData | CmdAck | RspType | RspData | RspAck | RspArbLost | RespSeq |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CMD_START | N/A | N/A | CMD_START | N/A | N/A | 0 | 0 |
| 2 | CMD_SEND | Addr + R/W | N/A | CMD_SEND | N/A | 1 | 0 | 0 |
| 3 | CMD_REC | N/A | 1 | CMD_REC | Data | N/A | 0 | 0 |
| 4 | CMD_REC | N/A | 0 | CMD_REC | Data | N/A | 0 | 0 |
| 5 | CMD_STOP | N/A | N/A | CMD_STOP | N/A | N/A | 0 | 0 |

#### Two Byte Write

- CMD_START – send start condition

- CMD_SEND – send address byte (slave responds with ACK)

- CMD_SEND – send data (slave responds with ACK)

- CMD_SEND – send data (slave responds with NACK)

- CMD_STOP – send stop condition

| Order | CmdType | CmdData | CmdAck | RspType | RspData | RspAck | RspArbLost | RespSeq |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CMD_START | N/A | N/A | CMD_START | N/A | N/A | 0 | 0 |
| 2 | CMD_SEND | Addr + R/W | N/A | CMD_SEND | N/A | 1 | 0 | 0 |
| 3 | CMD_SEND | Data | N/A | CMD_SEND | N/A | 1 | 0 | 0 |
| 4 | CMD_SEND | Data | N/A | CMD_SEND | N/A | 0 | 0 | 0 |
| 5 | CMD_STOP | N/A | N/A | CMD_STOP | N/A | N/A | 0 | 0 |

#### One Byte Write followed by One Byte Read (with Repeated Start)

- CMD_START – send start condition

- CMD_SEND – send address byte (slave responds with ACK)

- CMD_SEND – send data (slave responds with ACK)

- CMD_REPSTART – Repeated start

- CMD_REC – receive data and send NACK

- CMD_STOP – send stop condition

| Order | CmdType | CmdData | CmdAck | RspType | RspData | RspAck | RspArbLost | RespSeq |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CMD_START | N/A | N/A | CMD_START | N/A | N/A | 0 | 0 |
| 2 | CMD_SEND | Addr + R/W | N/A | CMD_SEND | N/A | 1 | 0 | 0 |
| 3 | CMD_SEND | Data | N/A | CMD_REC | N/A | 1 | 0 | 0 |
| 4 | CMD_REPST. | N/A | N/A | CMD_REPST. | N/A | N/A | 0 | 0 |
| 4 | CMD_REC | N/A | 0 | CMD_REC | Data | N/A | 0 | 0 |
| 5 | CMD_STOP | N/A | N/A | CMD_STOP | N/A | N/A | 0 | 0 |

#### Arbitration Lost

- CMD_START – send start condition

- CMD_SEND – send address byte (slave responds with ACK)

- CMD_SEND – send data (arbitration is lost during this byte)

- CMD_REPSTART – Repeated start

  - This command is ignored (RespSeq=’1’) because it is illegal when the bus is not owned

| Order | CmdType | CmdData | CmdAck | RspType | RspData | RspAck | RspArbLost | RespSeq |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CMD_START | N/A | N/A | CMD_START | N/A | N/A | 0 | 0 |
| 2 | CMD_SEND | Addr + R/W | N/A | CMD_SEND | N/A | 1 | 0 | 0 |
| 3 | CMD_SEND | Data | N/A | CMD_REC | N/A | N/A | 1 | 0 |
| 4 | CMD_REPST. | N/A | N/A | CMD_REPST. | N/A | N/A | 0 | 1 |

### Example Waveform

The waveform below shows the very simplest transaction possible: transmitting an address only to probe if the slave is available. This simple transaction was chosen to keep the waveform as short as possible. The main focus is on the sequence of events, not on the I2C transaction.

Figure 24: psi_common_i2c_master: Address only transaction

## psi_common_axi_master_simple

### Description

This entity executes transactions requested through a simple command interface on an AXI bus according to all specifications. This entity includes FIFOs to buffer read- and write-data but not for the commands.

The user can request transaction of any size and they will get split automatically in order to not exceed AXI bust size and other limitations. The response is sent to the user when his whole command is executed (which may involve multiple AXI transactions).

For each command there are two operation modes:

- High Latency

  - The AXI-master only starts the command after sufficient data (write-case) or space (read-case) is available in the corresponding data FIFO

  - This ensures that commands can be executed without blocking the AXI bus.

  - This approach leads to more latency, since he user has to handle data before the command is sent.

- Low Latency

  - The AXI-master starts the transaction immediately, with no regard on FIFO states.

  - If the user logic cannot provide the data in-time, the AXI bus may get blocked.

  - This approach leads to lowest latency since the user logic can prepare the data on the fly without the transaction being delayed.

This entity does not handle unaligned transactions and word-width conversions. So the data- and AXI-width are the same and all commands must be aligned to that word-width. This is the reason for the term simple in the name of the entity.

Read and write logic are fully independent. So reads and writes can happen at the same time.

There is no required timing relationship between command and data signals. So for writes the user can provide write data before, after or together with the command.

Figure 25: psi_common_axi_master_simple: Block diagram

### Transaction Types

For simplicity, only burst transactions are shown. However, of course also single-word transactions are supported.

Note that for all examples, the maximum AXI burst length is assumed to be 4 (unusual low) for illustrative reasons.

Also not that latencies and delays may be drawn shorter than they actually are to keep the waveforms small. However, all relationship between signals are correct.

#### Write High-Latency

The example below shows a high latency burst read transaction.

Please read the description of all examples (10.3.2).

Figure 26: psi_common_axi_master_simple: High latency write

The waveform shows, that the write command (M_Axi_Aw*) is held back until all data for a burst (4 words) is in the FIFO. The command is then executed. The next command is executed immediately, because the data is already available when the first transaction completed.

The waveform also clearly shows, that the CmdWr_Rdy signals goes high as soon as all AXI-commands related to the user command are sent. However, at the time CmdWr_Rdy goes high, not all data is transferred yet. This only indicates that the next command can be applied and does not have any meaning for the currently processed signal.

The WrDone signal is pulsed as soon as the response of the last AXI transaction is received.

The waveform also clearly shows that a user command is split into two AXI transactions automatically and that the M_Axi_AwAddr and M_Axi_AwLen signals are chosen appropriately.

Since the waveform above only shows that the first transaction is delayed according to high-latency operation, a second figure is shown below that shows this behavior also for the second transaction.

Figure 27: psi_common_axi_master_simple: High latency write with delay for second transaction

#### Write Low-Latency

The example below shows a low latency burst read transaction.

Please read the description of all examples (10.3.2).

Figure 28: psi_common_axi_master_simple: Low latency write

The waveform shows, that in low latency operation, AXI commands are issued as soon as possible independently of the availability of data. Therefore both write commands are issued before even the data for the first one is in the FIFO.

The waveform also shows, that the M_AXI_W* bus is blocked temporarily (M_Axi_WValid low) due to the data not being available. This situation has a negative impact on the AXI bandwidth, so it shall be avoided usually.

To avoid stalling the AXI bus, it is possible to prefill the write data FIFO. To do so, the write command is sent after the first few data samples are already written into the FIFO. This allows using the FIFO to prevent the AXI bus from stalling.

Figure 29: psi_common_axi_master_simple: Low latency write with FIFO prefill

#### Read

The example below shows a burst read transaction.

Please read the description of all examples (10.3.2).

Figure 30: psi_common_axi_master_simple: Read transaction

Usually for reads there is enough space in the read FIFO. In this case, the user command directly triggers the transmission of the AXI-command on M_Axi_Ar*. After all AXI commands are sent, the FSM is ready for the next command.

If the slave is not able to continuously burst data, this is reflected on the read data output. However, a FIFO is present and can compensate this effect if reading of the data is started a few beats after availability of first data.

#### Read FIFO full with Low Latency

The example below shows a burst read transaction in low latency mode. In contrast to the example above, the read FIFO is assumed to be full when the user command is issued.

Please read the description of all examples (10.3.2).

Figure 31: psi_common_axi_master_simple: Read transaction, low latency

Beause the command is issued in low-latency mode, the AXI read command is issued immediately. Because the FIFO is full, M_Axi_RReady is low and the AXI bus is stalled. The pattern of space in the FIFO becoming available is visible on the AXI bus directly.

#### Read FIFO full with High Latency

The example below shows a burst read transaction in high latency mode. In contrast to the example above, the read FIFO is assumed to be full when the user command is issued.

Please read the description of all examples (10.3.2).

Figure 32: psi_common_axi_master_simple: Read transaction, high latency

Beause the command is issued in high-latency mode, the AXI read command is not issued until enough data is read from the FIFO in order for the command to complete in one burst. If this is the case, the AXI read command is issued and the transfer is completed in one burst with M_Axi_RValid high all the time.

This has the benefit of not blocking the AXI bus. In contrast to write commands, the high-latency mode does not lead to significantly more latency in the read-case because the user can still immediately read the first data after it was received. As a result, it is recommended to always execute read commands I the high-latency mode unless there is a very good reason for the low-latency mode.

### Generics

AxiAddrWidth_g				Width of the AXI address bus
AxiDataWidth_g				Width of the AXI data bus
AxiMaxBeats_g				Maximum number of beats in one AXI transaction. Values given by the 
 						AXI specification are 16 for AXI-3 and 256 for AXI-4. However, the user 
 						may choose any other number for scheduling reasons.
AxiMaxOpenTransactions_g		Maximum number of AXI commands (AW/AR-channel) to send before 
 						the first command is completed (outstanding transactions).
UserTransactionSizeBits _g		Number of bits used to specify the number of beats to transfer on the 
 						user command interface. This is the only limiting factor for the transfer 
 						size requested.
DataFifoDepth_g				Number of entries in the read/write data FIFOs
ImplRead_g				Implement read functionality (can be disabled to save resources)
ImplWrite_g				Implement write functionality (can be disabled to save resources)
RamBehavior_g				Block-RAM style (must match FPGA architecture)
 						“RBW”	Read before write
 						“WBR”	Write before read

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| M_Axi_Aclk | Input | 1 | Clock |
| M_Axi_Aresetn | Input | 1 | Reset (low active) |
| Write Command |  |  |  |
| CmdWr_Addr | Input | AxiAddrWidth_g | Address to start writing at (must be aligned) |
| CmdWr_Size | Input | UserTransactionSizeBits_g | Number of beats in the transfer |
| CmdWr_LowLat | Input | 1 | ‘1’ --> Low latency mode ‘0’ --> High latency mode |
| CmdWr_Vld | Input | 1 | AXI-S handshaking signal |
| CmdWr_Rdy | Output | 1 | AXI-S handshaking signal |
| Read Command |  |  |  |
| CmdRd_Addr | Input | AxiAddrWidth_g | Address to start reading at (must be aligned) |
| CmdRd_Size | Input | UserTransactionSizeBits_g | Number of beats in the transfer |
| CmdRd_LowLat | Input | 1 | ‘1’ --> Low latency mode ‘0’ --> High latency mode |
| CmdRd_Vld | Input | 1 | AXI-S handshaking signal |
| CmdRd_Rdy | Output | 1 | AXI-S handshaking signal |
| Write Data |  |  |  |
| WrDat_Data | Input | AxiDataWidth_g | Write data |
| WrDat_Be | Input | AxiDataWidth_g/8 | Byte enables for write data |
| WrDat_Vld | Input | 1 | AXI-S handshaking signal |
| WrDat_Rdy | Output | 1 | AXI-S handshaking signal |
| Read Data |  |  |  |
| RdDat_Data | Output | AxiDataWidth_g | Read data |
| RdDat_Vld | Output | 1 | AXI-S handshaking signal |
| RdDat_Rdy | Input | 1 | AXI-S handshaking signal |
| Response |  |  |  |
| Wr_Done | Output | 1 | Write command was completed successfully |
| Wr_Error | Output | 1 | Write command was completed but at least one transaction failed (AXI response from slave indicated an error) |
| Rd_Done | Output | 1 | Read command was completed successfully |
| Rd_Error | Output | 1 | Read command was completed but at least one transaction failed (AXI response from slave indicated an error) |
| AXI Master Interface |  |  |  |
| M_Axi_* | * | * | AXI signals, see AXI specification |

## psi_common_axi_master_full

### Description

This entity executes transactions requested through a simple command interface on an AXI bus according to all specifications. This entity includes FIFOs to buffer read- and write-data but not for the commands.

This this entity internally uses psi_common_axi_master_simple and works similarly Many generics are just forwarded to this component and the meaning of the low-latency function is the same. These topics are not explained in detail in this section, please refer to 10.3 for details.

For the case that the AXI data width is larger than the user data width, it is highly recommended to not use low-latency transfers. Due to the width conversion, the bandwidth is smaller than the maximum of AXI, so the bus would be blocked longer than required if low latency transfers are made.

The user can request transaction of any size and they will get split automatically in order to not exceed AXI bust size and other limitations. The response is sent to the user when his whole command is executed (which may involve multiple AXI transactions).

In contrast to the psi_common_axi_master_simple, this entity has the following additional features:

- Execution of unaligned and odd-sized transfers (alignment of the data according to AXI requirements)

- AXI data-width can be larger than user interface data width

- The transfer size is specified in bytes and not in beats (to allow uneven-length transfers)

Most of the logic in this entity is related to unaligned and odd-sized transfers. So if you only require an AXI data width that is larger than the user interface data width, you may consider using psi_common_axi_master along with psi_common_wconv_n2xn (with conversion) to achieve better performance with less resource utilization.

The psi_common_axi_master_full has some clock cycles of overhead for each command. As a result, large data transfers are efficient but the performance for very small transfers (only a few AXI bursts) is limited due to this overhead.

Read and write logic are fully independent. So reads and writes can happen at the same time.

There is no required timing relationship between command and data signals. So for writes the user can provide write data before, after or together with the command.

Figure 33: psi_common_axi_master_full: Block diagram

### Transaction Types

For simplicity, only burst transactions are shown. However, of course also single-word transactions are supported.

Note that latencies and delays may be drawn shorter than they actually are to keep the waveforms small. However, all relationship between signals are correct.

For all figures, an AXI-width of 32 bits and a user data width of 16 bits is assumed to keep the figures simple.

For simplicity reasons, only transfers that consist of one AXI transaction are shown. For the user interface, larger transactions (consisting of multiple AXI bursts) do behave exactly the same.

For all transactions, user data is right-aligned. The byte at the start address specified in the command is the LSB of the user input/output data.

#### Read Transaction

The example below shows a read transaction.

Please read the description of all examples (10.4.2).

Figure 34: psi_common_axi_master_full: Read transaction

The read command together with the address of the first used byte and the size of the data required in bytes is asserted. The psi_common_axi_master_full then calculates the word-aligned AXI start address (0x0200) and the number of AXI-beats required (3 --> M_Axi_ArLen=2) and asserts the AXI AR-command.

The received data is aligned correctly in order to have the first byte the user requested as LSB of the first RdDat word. Unused trailing bytes may have any value, they shall never be interpreted.

Note that the Rd_Done signal is asserted as soon as all data is read from AXI, even if the data was not yet consumed on the RdDat interface.

#### Write Transaction

The example below shows a read transaction.

Please read the description of all examples (10.4.2).

Note that a high-latency write (AXI commands only sent after data is in buffer) is shown in this figure.

Figure 35: psi_common_axi_master_full: Write transaction

In this example, the user provides some data before the command. This is perfectly fine and allowed. Then the write command together with the address of the first byte to write and the size of the data that must be written in bytes is asserted. The psi_common_axi_master_full then calculates the word-aligned AXI start address (0x0200) and the number of AXI-beats required (3 --> M_Axi_ArLen=2) and asserts the AXI AW-command.

The user data is automatically aligned to the alignment of the AXI bus. Unused bytes may have any value, which is fine because the corresponding strobe signal is set to low. The same applies to the user interface: unused trailing bytes can have any value and do not influence the transaction.

As soon as the write response from AXI is received, the Wr_Done signal is asserted.

### Generics

AxiAddrWidth_g				Width of the AXI address bus 
AxiDataWidth_g				Width of the AXI data bus
AxiMaxBeats_g*				Maximum number of beats in one AXI transaction. Values given by the 
 						AXI specification are 16 for AXI-3 and 256 for AXI-4. However, the user 
 						may choose any other number for scheduling reasons.
AxiMaxOpenTransactions_g		Maximum number of AXI commands (AW/AR-channel) to send before 
 						the first command is completed (outstanding transactions).
UserTransactionSizeBits _g		Number of bits used to specify the number of bytes to transfer on the 
 						user command interface. This is the only limiting factor for the transfer 
 						size requested.
DataFifoDepth_g				Number of entries in the read/write data FIFOs (user side)
AxiFifoDepth_g				Number of entries in the FIFOs inside the AXI interface
DataWidth_g				Width of the user data interface
ImplRead_g				Implement read functionality (can be disabled to save resources)
ImplWrite_g				Implement write functionality (can be disabled to save resources)
RamBehavior_g				Block-RAM style (must match FPGA architecture)
 						“RBW”	Read before write
 						“WBR”	Write before read

#### FIFO Parametrization

This section explains the most important points about parametrizing buffer sizes on the user side (DataFifoDepth_g) and on the AXI side (AxiFifoDepth_g).

The DataFifoDepth_g sets the depth of the FIFOs on the user interface as shown in the block diagram. For writes, this determines how much data can be accepted before a command is applied. The user side buffers have the width of the user-interface, so for writes, data in these buffers still must be converted to the AXI bus width (if AxiDataWith_g > DataWidth_g) and cannot be transmitted over AXI with ideal performance.

The AxiFifoDepth_g sets the depth of the FIFOs inside the AXI interface that have the same width as the AXI bus. These FIFOs can only be filled after a command is sent. On the other hand, the data buffered in these FIFOs can be sent over AXI with ideal performance, so this buffer shall be used to compensate effects due to the AXI bus being busy.

For reads, it does not matter much, in which FIFO the data is buffered since the bandwidth on the user-interface is always smaller or equal to the bandwidth of the AXI interface. As a result, the width-conversion does not lead to a bottleneck (like it is this case for writes).

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| M_Axi_Aclk | Input | 1 | Clock |
| M_Axi_Aresetn | Input | 1 | Reset (low active) |
| Write Command |  |  |  |
| CmdWr_Addr | Input | AxiAddrWidth_g | Address to start writing at (must be aligned) |
| CmdWr_Size | Input | UserTransactionSizeBits_g | Number of bytes in the transfer |
| CmdWr_LowLat | Input | 1 | ‘1’ --> Low latency mode ‘0’ --> High latency mode |
| CmdWr_Vld | Input | 1 | AXI-S handshaking signal |
| CmdWr_Rdy | Output | 1 | AXI-S handshaking signal |
| Read Command |  |  |  |
| CmdRd_Addr | Input | AxiAddrWidth_g | Address to start reading at (must be aligned) |
| CmdRd_Size | Input | UserTransactionSizeBits_g | Number of bytes in the transfer |
| CmdRd_LowLat | Input | 1 | ‘1’ --> Low latency mode ‘0’ --> High latency mode |
| CmdRd_Vld | Input | 1 | AXI-S handshaking signal |
| CmdRd_Rdy | Output | 1 | AXI-S handshaking signal |
| Write Data |  |  |  |
| WrDat_Data | Input | DataWidth_g | Write data (right-aligned) |
| WrDat_Vld | Input | 1 | AXI-S handshaking signal |
| WrDat_Rdy | Output | 1 | AXI-S handshaking signal |
| Read Data |  |  |  |
| RdDat_Data | Output | DataWidth_g | Read data (right-aligned) |
| RdDat_Vld | Output | 1 | AXI-S handshaking signal |
| RdDat_Rdy | Input | 1 | AXI-S handshaking signal |
| Response |  |  |  |
| Wr_Done | Output | 1 | Write command was completed successfully |
| Wr_Error | Output | 1 | Write command was completed but at least one transaction failed (AXI response from slave indicated an error) |
| Rd_Done | Output | 1 | Read command was completed successfully |
| Rd_Error | Output | 1 | Read command was completed but at least one transaction failed (AXI response from slave indicated an error) |
| AXI Master Interface |  |  |  |
| M_Axi_* | * | * | AXI signals, see AXI specification |

## psi_common_axi_slave_ipif

### Description

This entity implements a full AXI-4 slave interface that can be used to make custom IP-Cores accessible through AXI.

On the user interface (where the user code is attached), it supports using registers as well as access to synchronous memory (e.g. BRAMs). Burst are supported, also across the boundary between registers and memory range.

The limitations of this block are given below:

- It cannot be operated with memory only (at least 1 register must be used)

- The number of registers must be a power of two

- The latency of memory attached must be exactly one clock cycle

- AXI bus width is fixed to 32-bits

Especially the limitation of the memory latency to one clock cycle is suboptimal, since this prevents any additional pipelining in large IP-Cores.

For registers, this entity handles read/write registers completely independently. If readback of register values written via AXI should be possible, the user code must loop-back write values (o_reg_wdata) to read values (i_reg_rdata).

The memory range is placed in the memory map directly after the registers. Example: If 8 registers are implemented, the registers are at AXI addresses 0x00, 0x04, … 0x1C and memory starts at the AXI address 0x20.

The offset of the memory is removed internally. So in the example above, an access to the AXI address 0x24 (second memory cell) leads to the memory address (o_mem_addr) 0x04 because the offset of 0x20 is subtracted in the psi_common_axi_slave_ipif component.

### IP Interface transactions

Only burst transactions of length 4 are shown in the waveforms for simplicity reasons. Single word transactions behave the same as length 1 bursts.

For all waveforms, an implementation with 4 registers (NumReg_g = 4) is assumed. Hence the memory range starts at address 0x10.

#### Register Write

When a register is written, a pulse on the corresponding o_reg_wr signal is asserted together with the new data value.

Figure 36: psi_common_axi_slave_ipif: Register Write

#### Register Read

When a register is read, its value is sampled together with the pulse being applied on the corresponding o_reg_rd signal. Hence he o_reg_rd signal can for example be used to acknowledge reading from a FIFO.

Figure 37: psi_common_axi_slave_ipif: Register Read

#### Memory Write

In this example, data in the AXI-addresses 0x12 … 0x1D is written. Since the example assumes four registers (addresses 0x00 … 0x0F), this translates to memory addresses 0x02 … 0x0D on the user interface, because the memory offset of 0x10 is subtracted internally.

Figure 38: psi_common_axi_slave_ipif: Memory Write

#### Memory Read

In this example, data in the AXI-addresses 0x10 … 0x1F is read. Since the example assumes four registers (addresses 0x00 … 0x0F), this translates to memory addresses 0x01 … 0x0F on the user interface, because the memory offset of 0x10 is subtracted internally.

The example also nicely shows, that read data must be applied after exactly one clock cycle.

Figure 39: psi_common_axi_slave_ipif: Memory Read

#### Write over Register/Memory Boundary

In this example, four data words are written to the addresses 0x08 … 0x17. This includes two registers and two memory locations. Note that the register and memory interfaces are not delay compensated, therefore the first memory access happens at the same time as the last register access.

Figure 40: psi_common_axi_slave_ipif: Write over Register/Memory Boundary

### Generics

NumReg_g			Number of registers to implement 
ResetVal_g			Reset values for registers. The size of the array passed does not have to match 
 				NumReg_g, if it does not, the reset values are applied to the first N registers and the 
 				other registers are reset to zero.
UseMem_g			True = use memory interface, False = use registers only
AxiIdWidth_g		Number of bits used for the AXI ID signals
AxiAddrWidth _g	Number of AXI address bits supported

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| s_axi_aclk | Input | 1 | Clock |
| s_axi_aresetn | Input | 1 | Reset (low active) |
| Register Interface |  |  |  |
| o_reg_rd | Output | NumReg_g | Read-pulse for each register |
| i_reg_rdata | Input | NumReg_g x 32 | Register read values |
| o_reg_wr | Input | NumReg_g | Write-pulse for each register |
| o_reg_wdata | Input | NumReg_g x 32 | Register write values |
| Memory Interface |  |  |  |
| o_mem_addr | Output | AxiAddrWidth_g | Memory address |
| o_mem_wr | Output | 4 | Memory byte write enables (one signal per byte) |
| o_mem_wdata | Output | 32 | Memory write data |
| i_mem_rdata | Input | 32 | Memory read data Must be valid one clock cycle after o_mem_addr |
| AXI Slave Interface |  |  |  |
| s_axi_* | * | * | AXI signals, see AXI specification |

# Miscellaneous

## psi_common_delay

### Description

This component is an efficient implementation for delay chains. It uses FPGA memory resources (Block-RAM and distributed RAM resp. SRLs) for implementing the delays (instead of many FFs). The last delay stage is always implemented in FFs to ensure good timing (RAM outputs are usually slow).

One Problem with using RAM resources to implement delays is that they don’t have a reset, so the content of the RAM persists after resetting the logic. The psi_common_delay entity works around this issue by some logic that ensures that any persisting data is replaced by zeros after a reset. The replacement is done at the output of the psi_common_delay, so no time to overwrite memory cells after a reset is required and the entity is ready to operate on the first clock cycle after the reset.

If the delay is implemented using a RAM, the behavior of the RAM (read-before-write or write-before-read) can be selected to allow efficient implementation independently of the target technology.

### Generics

Width_g			Width of the data to delay
Delay_g			Number of delay taps
Resource_g		“AUTO” (default) 	automatically use SRL or BRAM according to BramThreshold_g
 				“BRAM” 		use Block RAM to implement the delay taps
 				“SRL”			use SRLs (LUTs used as shift registers) to implement the delay 
 							taps
BramThreshold_g	This generic controls the resources to use for the delay taps in case Resource_g = 
 				“AUTO”. SRLs are used if Delay_g < BramThreshold_g. Otherwise BRAMs are used.
RstState_g			True 		Persisting memory content is replaced by zeros after reset
 				False		Persisting memory content is visible at output after reset (less resource 
 						usage)
RamBehavior_g		“RBW”		Read-before-write implementation
 				“WBR”		Write-before-read implementation
 				This generic is only used if a BRAM is used for the delay.

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Rst | Input | 1 | Reset (high active) |
| Data |  |  |  |
| Vld | Input | 1 | InData valid (clock enable for shift register) |
| InData | Input | Width_g | Data input |
| OutData | Output | Width_g | Data output |

## psi_common_pl_stage

### Description

This component implements a pipeline stage that supports full AXI-S handshaking (including the handling of back-pressure). The pipeline breaks any combinatorial paths on all lines (Rdy, Vld and Data). So not only the forward signals Vld and Data are registered but also Rdy. This is important since long combinatorial paths are common to occur on the Rdy signal (it is often handled combinatorial).

Correct handling of the Rdy signal requires some additional resources. Therefore the handling of Rdy can be disabled using a generic to reduce resource usage if back-pressure must not be handled.

### Generics

Width_g			Width of the data signal
UseRdy_g			True 		Backpressure is handled (Rdy is used and pipelined)
 				False		Backpressure is not handled (Rdy is not connected at all in this case)

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Rst | Input | 1 | Reset (high active) |
| Input |  |  |  |
| InVld | Input | 1 | AXI-S handshaking signal |
| InRdy | Output | 1 | AXI-S handshaking signal |
| InData | Input | Width_g | Data input |
| OutVld | Output | 1 | AXI-S handshaking signal |
| OutRdy | Input | 1 | AXI-S handshaking signal |
| OutData | Output | Width_g | Data output |

## psi_common_multi_pl_stage

### Description

This component implements allows easily adding multiple pipeline stages to a signal path and maintain full AXI-S handshaking including back-pressure. It does so by chaining multiple psi_common_pl_stage (see 11.2) entities.

### Generics

Width_g			Width of the data signal
UseRdy_g			True 		Backpressure is handled (Rdy is used and pipelined)
 				False		Backpressure is not handled (Rdy is not connected at all in this case)
Stages_g			Number of pipeline stages

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| Clk | Input | 1 | Clock |
| Rst | Input | 1 | Reset (high active) |
| Input |  |  |  |
| InVld | Input | 1 | AXI-S handshaking signal |
| InRdy | Output | 1 | AXI-S handshaking signal |
| InData | Input | Width_g | Data input |
| OutVld | Output | 1 | AXI-S handshaking signal |
| OutRdy | Input | 1 | AXI-S handshaking signal |
| OutData | Output | Width_g | Data output |

## psi_common_ping_pong

### Description

This component implements a ping pong buffer mechanism around a single RAM block for multiple channels. It allows to stream data continuously in moving data back and forth at different memory space. While a buffer ‘Ping’ is written the other ‘Pong’ can be read. An interrupt is delivered when the buffer writing address swap, it gives the start indication to read.

The memory split between “buffers” which correspond to channels is defined as follow: the memory space necessary for one single channel will be extracted from the number of samples to be transferred and will be ceiled to its power of 2. This means gaps may occur if one choose to forward 500 samples a memory fragmentation of 12 samples per canal is to be expected. This choice has been to ease the genericity of the component and its implementation. At the read port it is possible to select channel independent from sample. The ping pong component allows either receiving data in parallel either as time division multiplexed. Fig. 39 gives an overview of the TDM behavior whereas the fig. 40 shows the TDM mode.

If user wants to use a single channel then the parallel mode is required. Beware; data sample frequency ratio compared to the clock cannot be higher than the number of channel. It is not feasible to several channels simultaneously, in other words only one sample per ‘number of channels’ clock cycles. An associated test bench helps to determinate proper parameters.

Figure 41: psi_common_ping_pong TDM mode

Figure 42: psi_common_ping_pong parallel

Both figures shows hypothetical behaviors for ping pong buffer and the following convention applies:

- C: channel

- S: sample

- C0S1: Channel 0 & sample 1

### Generics

ch_nb_g			Number of Channel
sample_nb_g		Number of sample to store

dat_length_g		Vector width per channel

tdm_g			True 			TDM data flow mode
 				False			Parallel mode

ram_behavior_g		“RBW”		Read-before-write implementation
 				“WBR”		Write-before-read implementation

rst_pol_g			Reset polarity

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| clk_i | Input | 1 | Clock processing |
| rst_i | Input | 1 | Reset processing |
| Input |  |  |  |
| dat_i | Input | Generic | Width depends on the mode - parallel: ch_nb_g* dat_length_g - tdm: dat_length_g |
| str_i | Input | 1 | AXI-S handshaking signal |
| Memory interface |  |  |  |
| mem_irq_o | Output | 1 | Interrupt signal high for one clock cycle that indicate that buffer swapped |
| mem_clk_i | Input | 1 | Clock memory |
| mem_addr_ch_i | Input | Log2ceil(ch_nb_g) | Address corresponding to channel (MSB) - |
| mem_addr_spl_i | Input | Log2ceil(sample_nb_g) | Address corresponding to sample (LSB) - |
| mem_dat_o | Output | dat_length_g | Data output |

## psi_common_delay_cfg

### Description

This component is slightly the same as the psi_common_delay (cf. §11.1) but it is configurable during runtime. It allows setting the delay by a register. The architecture is based on block RAM and a SRL an overview is shown on next figure.

Since the latency is set by 3 registers to write, to read from block ram and to output the value a small shift register is implemented to output value when delay is inferior or equal to 3.

The delay value 0 isn’t covered it will by default have a minimum of one clock cycle delay.

A generic allows to hold the value when a change is done to increase delay, this avoid having transient for example. A counter is launched to compute the delay difference and read address is hold during this time. This behavior can be skipped.

Figure 43: psi_common_delay_cfg: Hold behavior & pseudo-architecture

### Generics

Width_g			Width of the data to delay

MaxDelay_g		Number of delay taps (nb: The RAM size will be the next power of 2 of this value)

RstPol_g			Number of delay taps
RamBehavior_g		“RBW”		Read-before-write implementation
 				“WBR”		Write-before-read implementation

Hold_g		True	While a delay parameter value increase, it holds the last value that where generated and as soon as the delay has been compensated it output the expected value

False	output old value that where written previously into the RAM when the Delay excursion occurred

### Interfaces

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| clk_i | Input | 1 | Clock |
| rst_i | Input | 1 | Reset (polarity set by generic) |
| Data |  |  |  |
| str_i | Input | 1 | InData valid (clock enable for shift register) |
| dat_i | Input | Width_g | Data input |
| del_i | Input | Log2ceil(MaxDelay_g) | Number of delay taps |
| dat_o | Output | Width_g | Data output |

## psi_common_trigger_analog

### Description

This component can be used to generate a single clock pulse when its input value overpass a certain threshold.

The trigger generation can be continuous, hence a pulse is generated every time the condition is satisfied or single, and the trigger generator must be re-armed after every generated pulse.

The pulse is generated if the value represented in a (signed or unsigned) std_logic_vector crosses a threshold value decided by the user, the sensitive edge can be rising, falling or both.

Figure 44 psi_common_trigger_generator analog trigger type selected

NOTE: once the selected condition is satisfied, the trigger pulse is generated with a two clock cycle delay. This is due to the internal logic structure and it can be compensated externally by the user.

### Generics

analog_input_number_g	Number of analog trigger input

analog_input_width_g		Analog trigger input signals width

analog_trg_signed_g		Analog trigger input signals are signed

rst_pol_g				Reset polarity

### Interfaces

| Signal | Direction | Type | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| InClk | Input | std_logic | Clock |
| InRst | Input | std_logic | Reset (high active) |
| InTrgModeCfg | Input | std_logic_vector (0 downto 0) | Trigger mode: 0:Continuous 1:Single |
| InTrgArmCfg | Input | std_logic | Arm or Dis-arm the trigger mechanism. The logic is sensitive to the rising edge of this signal. |
| InTrgEdgeCfg | Input | std_logic_vector (1 downto 0) | Trigger edge direction: 00:both edges are disabled 01:only falling edge is enabled 10:only rising edge is enabled 11:both edges are enabled |
| Input |  |  |  |
| InTrgAnalogSourceCfg | Input | std_logic_vector(log2ceil(analog_input_number_g)-1 downto 0) | Select which InAnalogTrg use as input |
| InAnalogThTrg | Input | std_logic_vector (analog_input_width_g - 1 downto 0 | Threshold value if InTrgTypeCfg is 01 |
| InAnalogTrg | Input | std_logic_vector(analog_input_number_g * analog_input_width_g - 1 downto 0) | Analog input values |
| InExtDisarm | Input | std_logic | This input disarm the trigger without generating the OutTrigger pulse. |
| Output |  |  |  |
| OutTrgIsArmed | Output | std_logic | This signal is 1 if the trigger mechanism is armed |
| OutTrigger | Output | std_logic | Trigger output |

## psi_common_trigger_digital

### Description

This component can be used to generate a single trigger pulse when a rising and/or a falling edge of a std_logic signal is detected at the input.

The trigger generation can be continuous, hence a pulse is generated every time the condition is satisfied or single, and the trigger generator must be re-armed after a pulse is generated in this case.

Figure 45 psi_common_trigger_generator digital trigger type selected

NOTE: once the selected condition is satisfied, the trigger pulse is generated with a one clock cycle delay. This is due to the internal logic structure and it can be compensated externally by the user.

### Generics

digital_input_number_g	Number of digital trigger input

rst_pol_g				Reset polarity

### Interfaces

| Signal | Direction | Type | Description |
| --- | --- | --- | --- |
| Control Signals |  |  |  |
| InClk | Input | std_logic | Clock |
| InRst | Input | std_logic | Reset (high active) |
| InTrgModeCfg | Input | std_logic_vector (0 downto 0) | Trigger mode: 0:Continuous 1:Single |
| InTrgArmCfg | Input | std_logic | Arm or Dis-arm the trigger mechanism. The logic is sensitive to the rising edge of this signal. |
| InTrgEdgeCfg | Input | std_logic_vector (1 downto 0) | Trigger edge direction: 00:both edges are disabled 01:only falling edge is enabled 10:only rising edge is enabled 11:both edges are enabled |
| Input |  |  |  |
| InTrgDigitalSourceCfg | Input | std_logic_vector(log2ceil(digital_input_number_g)-1 downto 0) | InDigitalTrg bit index used as trigger input |
| InDigitalTrg | Input | std_logic_vector(digital_input_number_g - 1 downto 0) | Input vector |
| InExtDisarm | Input | std_logic | This input disarm the trigger without generating the OutTrigger pulse. |
| Output |  |  |  |
| OutTrgIsArmed | Output | std_logic | This signal is 1 if the trigger mechanism is armed |
| OutTrigger | Output | std_logic | Trigger out put |